CPU

Processor Organization:

1. Register based CPU organization

2. Accumulator based CPU organization

3. Stack based CPU organization

Accumulator based CPU organization:

Stack Based CPU organization:

Instruction Format:

1. Three address instruction format

→ MUL R1 ,A ,B  // R1 ← M[A] * M[B]

2. Two address instruction format

→ MOV R1 ,A   // R1← M[A]

→ MUL R1 ,B   // R1 ← R1 * M[B]

3. One address instruction format

→ MUL R1       // AC ← AC * R1

4. Zero address instruction format [stack based CPU organization]

→ The expression (A*B)+(C*D)  in zero address format will be as follows:

|  |  |  |  | D |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | B |  | C | C | C*D |  |
| A | A | A*B | A*B | A*B | A*B | (A*B)+(C*D) |

*each column is a step and step progresses from left to right.

Program:

PUSH A

PUSH B

MUL

PUSH C

PUSH D

MUL

ADD

POP X

* to write this program convert expression to postfix notation.

Addressing Modes:

The way of specifing operand in an instruction.

Instruction = opcode + operand

- Immediate → MVI A , 05H

- Implied → CMA

- Direct → LDA 2000H

- Indirect → ILDA 1000H (Address of address of operand)

- Register → ADD B

- Register Indirect → ADD M (M contains address of operand)

- Displacement

  - Relative → EA = address + PC (EA=effective address)

  - Index → EA = address + IR (IR=index register)

  - Base → EA = address + CS (CS=code segment)

  - Base + Index → EA = address + CS + DI (DS=destination index)

- Auto Increment / Decrement