Pipelining

It is the process of dividing sequencial process and parallelizing it in a processor.

Parallel Processing:

Flyn’s classification of parallel processing:

- Single instruction stream single data stream

- Single instruction stream multiple data stream

- Multiple instruction stream single data stream

- Multiple instruction stream muliple data stream

Single instruction stream single data stream

Single instruction stream multiple data stream

Multiple instruction stream single data stream

Multiple instruction stream muliple data stream

Speed-up Eqn:

Lets suppose there are “n” operations each operation takes “tn” time to complete. For n operations to complete:

total time = n * tn

Consider “K” segment pipeline. Each segment takes tp time to operate. For 1st operation to complete it will take k*tp time to complete. For remaining n-1 tasks, it will take

(n-1)*tp time.

So total time = k*tp + (n-1)*tp

= (k+n-1)*tp

Speed-up = time for non-pipeline/time for pipelined system

= n*tn/(k+n-1)tp

Arithmetic Pipeline:

Floating point addition and subtraction:

x = 2.5  x107

+ y = 3.36x106

2.836x107

Steps:

  - Compare exponent

  - Align mantissa (choosing larger exponent)

  - Add / Subtract mantissa

  - Normalize mantissa